IoT Smart Locker Security System Using ESP32, Blynk App, Servo Motor and Email Alert

You will not use touch sensor and not use OLED.

Instead:

Password entry → Blynk App
Status display → Blynk App
Lock opening → Servo motor
Alert message → Email
Controller → ESP32

Final Components

| Component | Purpose |
| --- | --- |
| ESP32 | Main controller |
| Servo motor | Lock/unlock mechanism |
| Magnetic door sensor | Detects forced opening |
| Buzzer | Alarm |
| Blynk app | Enter password and show status |
| WiFi | Internet connection |
| Email SMTP | Sends security alert |

Working

- User opens Blynk app.

- User enters password in app.

- ESP32 receives password through WiFi.

- If password is correct:

  - Servo motor unlocks the locker.

  - Blynk shows Access Granted / Locker Unlocked.

- If password is wrong:

  - Blynk shows Wrong Password.

  - Wrong attempt count increases.

- If password is wrong 3 times:

  - Buzzer turns ON.

  - Email alert is sent.

- If locker is forcefully opened:

  - Door sensor detects it.

  - Buzzer turns ON.

  - Email alert is sent.

Updated Block Diagram

Blynk App
 Password Entry + Status Display
              ↓
            WiFi
              ↓
            ESP32
       ↓       ↓       ↓
   Servo    Buzzer   Door Sensor
       ↓
 Locker Lock/Unlock

If wrong password / forced open
              ↓
          Email Alert

Blynk App Setup

Create a new Blynk project and add these widgets.

| Widget | Virtual Pin | Use |
| --- | --- | --- |
| Text Input | V0 | Enter password |
| Button | V1 | Submit password |
| Label / Value Display | V2 | Show locker status |
| LED widget | V3 | Lock status |
| Button | V4 | Lock manually |

Pin Connections

Servo Motor

| Servo Wire | ESP32 |
| --- | --- |
| Red | 5V |
| Brown/Black | GND |
| Orange/Yellow | GPIO 18 |

Use external 5V supply for servo if possible. Connect external supply GND with ESP32 GND.

Magnetic Door Sensor

| Door Sensor | ESP32 |
| --- | --- |
| One pin | GPIO 19 |
| Other pin | GND |

Buzzer

| Buzzer | ESP32 |
| --- | --- |
| Positive | GPIO 23 |
| Negative | GND |

Email Alert Message

SECURITY ALERT: Smart Locker Access WarningSECURITY ALERT: Smart Locker Access Warning

Security Alert!

Suspicious activity has been detected on the smart locker.

Alert Type: Wrong password attempts / Forced locker opening

Please check the locker immediately.

Device: IoT Smart Locker Security System

Required Arduino Libraries

Install these libraries:

Blynk
ESP Mail Client
ESP32Servo

What You Need to Change in Code

Change these values:

#define BLYNK_TEMPLATE_ID "YOUR_TEMPLATE_ID"
#define BLYNK_TEMPLATE_NAME "Smart Locker"
#define BLYNK_AUTH_TOKEN "YOUR_BLYNK_AUTH_TOKEN"

char ssid[] = "YOUR_WIFI_NAME";
char pass[] = "YOUR_WIFI_PASSWORD";

#define AUTHOR_EMAIL "your_email@gmail.com"
#define AUTHOR_PASSWORD "your_gmail_app_password"
#define RECIPIENT_EMAIL "receiver_email@gmail.com"

And password here:

String correctPassword = "1234";

You can change "1234" to your own locker password.